ROOF ASSESSMENT CHECKLIST

Free companion tool to Chapter 3: Panels and Placement

Property: ________________________________________      Date: __________

Use one column per roof face that could take panels. You'll need a phone compass app, a tape measure and a camera.

1. Roof faces

|  | Face A | Face B | Face C |
| --- | --- | --- | --- |
| Where is it? (e.g. front, above garage) |  |  |  |
| Direction it faces | ☐ Towards equator ☐ 45° off equator ☐ East ☐ West ☐ Away ☐ Flat | ☐ Towards equator ☐ 45° off equator ☐ East ☐ West ☐ Away ☐ Flat | ☐ Towards equator ☐ 45° off equator ☐ East ☐ West ☐ Away ☐ Flat |
| Compass bearing | ______ ° | ______ ° | ______ ° |
| Pitch (slope) | ☐ Flat (0–5°) ☐ Gentle (5–20°) ☐ Medium (20–35°) ☐ Steep (35°+) | ☐ Flat (0–5°) ☐ Gentle (5–20°) ☐ Medium (20–35°) ☐ Steep (35°+) | ☐ Flat (0–5°) ☐ Gentle (5–20°) ☐ Medium (20–35°) ☐ Steep (35°+) |
| Usable area (clear of edges, vents, skylights) | ___ m × ___ m = _____ m² | ___ m × ___ m = _____ m² | ___ m × ___ m = _____ m² |
| Approx. panels (area ÷ 2.7 m²) | _____ panels | _____ panels | _____ panels |
| Roof covering | ☐ Metal sheeting ☐ Concrete tile ☐ Clay tile ☐ Flat concrete ☐ Other | ☐ Metal sheeting ☐ Concrete tile ☐ Clay tile ☐ Flat concrete ☐ Other | ☐ Metal sheeting ☐ Concrete tile ☐ Clay tile ☐ Flat concrete ☐ Other |

2. Other places for panels

| Option | Area | Notes (shade, access, approvals) |
| --- | --- | --- |
| ☐ Ground-mount frame | _____ m² |  |
| ☐ Carport (a double carport gives about 30–35 m²) | _____ m² |  |
| ☐ Pergola | _____ m² |  |

3. Roof condition

Check from outside and inside the roof space. Any Yes means getting it checked before panels go on (Chapter 3, section 3.6).

|  | Yes | No |  | Yes | No |
| --- | --- | --- | --- | --- | --- |
| Sagging roofline, ridge or gutters | ☐ | ☐ | Damaged, rotten or termite-affected timbers | ☐ | ☐ |
| Rust, holes or loose fixings in sheeting | ☐ | ☐ | Roof due for repaint or replacement within 5–10 years | ☐ | ☐ |
| Cracked, broken or brittle tiles | ☐ | ☐ | Tall, exposed building, or high-wind area | ☐ | ☐ |
| Water stains on ceilings, or leaks | ☐ | ☐ | Tilting frames or ballast planned on a flat roof | ☐ | ☐ |

4. Summary

| Total usable area | _____ m² | Panels needed (PV Calculator) | _____ panels |
| --- | --- | --- | --- |
| Panels that fit (area ÷ 2.7 m²) | _____ panels | Room to spare, or shortfall | _____ panels |
| Array size that fits (panels × 0.6 kW) | _____ kW | Room for 20–30% extra panels? | ☐ Yes ☐ No |

Panel areas assume a modern 600 W panel of about 2.38 × 1.13 m. Leave a gap from roof edges as your installer and local rules require.

SHADING LOG

Roof Assessment Checklist, page 2

At each time, look at each roof face and mark: 0 = in full sun,  ½ = partly shaded,  1 = fully shaded. Shadows are much longer in winter, so fill in both columns if you can. If you can't wait for winter, remember the winter sun sits lower and further towards the equator.

| Time | Face A summer | Face A winter | Face B summer | Face B winter | Face C summer | Face C winter |
| --- | --- | --- | --- | --- | --- | --- |
| 7 am |  |  |  |  |  |  |
| 8 am |  |  |  |  |  |  |
| 9 am |  |  |  |  |  |  |
| 10 am |  |  |  |  |  |  |
| 11 am |  |  |  |  |  |  |
| 12 noon |  |  |  |  |  |  |
| 1 pm |  |  |  |  |  |  |
| 2 pm |  |  |  |  |  |  |
| 3 pm |  |  |  |  |  |  |
| 4 pm |  |  |  |  |  |  |
| 5 pm |  |  |  |  |  |  |

What causes the shade?     ☐ Trees   ☐ Chimney   ☐ Neighbouring building   ☐ Satellite dish or vent pipe   ☐ Overhead cables   ☐ Other: __________

Sketch your roof

Draw the roof faces, mark north (or south), and sketch anything that casts a shadow: trees, chimneys, neighbouring buildings.